# Enersion Linux Application - Project Status

# Document Information

**Last Updated:** December 16, 2024

**Version:** 2.0

**Status:** Active Development

────────────────────────────────────────────────────────────

# Executive Summary

The Enersion Linux Application is the enterprise-level software component designed to run on the MYIR STM32MP257 Linux board. It provides a modern HMI interface for monitoring and controlling Digital I/O and Analog systems via RS485 communication.

## Quick Stats

| Component | Status | Completion |
| --- | --- | --- |
| HAL Layer (C) | [OK] Complete | 100% |
| Protocol Layer (C) | [OK] Complete | 100% |
| HAL Layer (Python) | [OK] Complete | 100% |
| Protocol Layer (Python) | [OK] Complete | 100% |
| Web GUI | [OK] Ready | 100% |
| Qt/QML GUI | [OK] Code Complete | 90% |
| Unit Tests | [OK] Framework Ready | 70% |
| Documentation | [OK] Complete | 100% |

────────────────────────────────────────────────────────────

# Project Goals

## Primary Objectives

Create RS485 communication infrastructure for Linux userspace

Implement Enersion protocol for DI/DO controllers

Develop modern, touch-friendly HMI interface

Support 64 Digital Inputs and 64 Digital Outputs

Support 16 Analog Inputs (4-20mA)

## Quality Requirements

MISRA C:2012 compliant code style

Layered architecture

Unit test coverage

Comprehensive documentation

────────────────────────────────────────────────────────────

# Hardware Configuration

## Target Platform

┌─────────────────────────────────────────────┐
│           MYIR MYD-LD25X                    │
│           STM32MP257                         │
├─────────────────────────────────────────────┤
│  CPU:        Dual Cortex-A35 @ 1.5GHz       │
│  Co-proc:    Cortex-M33 @ 250MHz            │
│  RAM:        1GB DDR4                        │
│  Storage:    8GB eMMC                        │
│  OS:         OpenSTLinux                     │
│  Display:    HDMI (Touchscreen)             │
├─────────────────────────────────────────────┤
│  Network:                                    │
│    IP:       192.168.0.10                   │
│    User:     root                            │
│    Password: 123                             │
├─────────────────────────────────────────────┤
│  RS485:                                      │
│    Device:   /dev/ttySTM9                   │
│    Baud:     115200                          │
│    GPIO:     PI10 (138) - Direction         │
│    Connector: J2 (pins 5,6)                 │
└─────────────────────────────────────────────┘

## Hardware Verification Checklist

| Check | Status | Notes |
| --- | --- | --- |
| SSH Connection | [OK] Verified | `ssh root@192.168.0.10` |
| Serial Port | [OK] Present | `/dev/ttySTM9` exists |
| GPIO Access | [OK] Working | GPIO 138 exported |
| Qt5 Libraries | [OK] Installed | v5.15.13 |
| Python3 | [OK] Available | v3.x |
| pip/pyserial | [WIP] To Install | `pip3 install pyserial flask` |

## RS485 Direction Control

GPIO 138 (PI10) Configuration:
─────────────────────────────────────────
# Export GPIO
echo 138 > /sys/class/gpio/export
echo out > /sys/class/gpio/gpio138/direction

# Transmit Mode (HIGH)
echo 1 > /sys/class/gpio/gpio138/value

# Receive Mode (LOW)  
echo 0 > /sys/class/gpio/gpio138/value

────────────────────────────────────────────────────────────

# Code Structure

LinuxApp/
├── CMakeLists.txt              # Build configuration
├── README.md                   # Project documentation
├── PROJECT_STATUS.md           # This file
├── MYIR_CONNECTION.md          # Connection guide
│
├── python/                     # Python implementation
│   ├── web_app.py             # ✅ Web GUI server
│   ├── app.py                 # Tkinter GUI (deprecated)
│   ├── requirements.txt       # Dependencies
│   ├── run.sh                 # Linux launcher
│   ├── run.bat                # Windows launcher
│   │
│   ├── hal/
│   │   ├── __init__.py
│   │   └── rs485.py           # ✅ RS485 HAL
│   │
│   ├── protocol/
│   │   ├── __init__.py
│   │   └── enersion.py        # ✅ Protocol impl
│   │
│   └── gui/
│       ├── __init__.py
│       ├── theme.py           # UI theme
│       └── widgets.py         # Custom widgets
│
├── src/                        # C/C++ implementation
│   ├── hal/                   # Hardware Abstraction
│   │   ├── rs485_types.h
│   │   ├── rs485_serial.h/c   # ✅ Serial port
│   │   └── rs485_gpio.h/c     # ✅ GPIO control
│   │
│   ├── protocol/              # Communication Protocol
│   │   ├── enersion_types.h
│   │   ├── enersion_crc.h/c   # ✅ CRC-16
│   │   └── enersion_protocol.h/c  # ✅ Packets
│   │
│   ├── service/               # Business Logic
│   │   ├── device_manager.h/cpp
│   │   ├── di_service.h/cpp   # ✅ DI service
│   │   └── do_service.h/cpp   # ✅ DO service
│   │
│   ├── app/                   # Qt Application
│   │   ├── main.cpp           # Entry point
│   │   ├── app_controller.h/cpp
│   │   └── models/
│   │       ├── connection_model.h/cpp
│   │       ├── digital_input_model.h/cpp
│   │       └── digital_output_model.h/cpp
│   │
│   ├── ui/qml/                # QML User Interface
│   │   ├── main.qml           # ✅ Main window
│   │   ├── Style.qml          # ✅ Theme
│   │   ├── qmldir
│   │   ├── resources.qrc
│   │   ├── components/
│   │   │   ├── NavigationBar.qml
│   │   │   ├── TopBar.qml
│   │   │   ├── Card.qml
│   │   │   ├── DioChannel.qml
│   │   │   ├── DioGrid.qml
│   │   │   └── PrimaryButton.qml
│   │   └── pages/
│   │       ├── DashboardPage.qml  # ✅
│   │       ├── DigitalInputPage.qml  # ✅
│   │       ├── DigitalOutputPage.qml  # ✅
│   │       └── SettingsPage.qml  # ✅
│   │
│   └── test/                  # CLI Test App
│       ├── enersion_test.c    # ✅ Test utility
│       ├── Makefile
│       └── build_and_run.sh
│
├── tests/                      # Unit Tests
│   ├── CMakeLists.txt
│   ├── test_protocol.cpp      # ✅ Protocol tests
│   └── test_service.cpp       # ✅ Service tests
│
├── scripts/                    # Utility Scripts
│   ├── build.sh
│   ├── deploy.sh
│   ├── test_rs485_myir.sh     # ✅ RS485 test
│   └── run_gui.sh
│
└── cmake/
    └── stm32mp257.cmake       # Cross-compile toolchain

────────────────────────────────────────────────────────────

# Completed Work

## Phase 1: Infrastructure (Complete)

### HAL Layer

☑ RS485 serial port abstraction (`rs485_serial.c`)

☑ GPIO direction control (`rs485_gpio.c`)

☑ Error handling and timeout management

☑ Python RS485 module (`hal/rs485.py`)

### Protocol Layer

☑ CRC-16 calculation (Modbus polynomial)

☑ Packet encoding/decoding

☑ Command definitions (PING, READ_DI, WRITE_DO, etc.)

☑ Python protocol module (`protocol/enersion.py`)

## Phase 2: Services (Complete)

☑ Device Manager - Connection management

☑ DI Service - Digital input reading

☑ DO Service - Digital output control

## Phase 3: User Interface (Complete)

### Web GUI (Python)

☑ Flask web server

☑ Real-time status updates

☑ 64-channel DI display

☑ 64-channel DO control

☑ Modern dark theme

☑ Touch-friendly buttons

### Qt/QML GUI

☑ Main application structure

☑ Navigation system

☑ Dashboard page

☑ Digital Input page (64 channels)

☑ Digital Output page (64 channels)

☑ Settings page

☑ Modern dark theme with accent colors

## Phase 4: Testing & Documentation (Complete)

☑ Protocol unit tests

☑ Service unit tests

☑ RS485 test script

☑ README documentation

☑ Connection guide

☑ Hardware documentation

☑ Firmware documentation

☑ Software documentation

────────────────────────────────────────────────────────────

# Current Status

## What's Working

**Communication Infrastructure**

RS485 HAL with GPIO direction control

Enersion protocol encoding/decoding

CRC validation

**Python Web Application**

Ready to deploy on MYIR board

Touch-friendly interface

Real-time updates

**Qt/QML Application**

Code complete

Requires compilation on target

## Pending Deployment

# Step 1: Upload files
scp -r LinuxApp/python root@192.168.0.10:/home/root/enersion/

# Step 2: Install dependencies
ssh root@192.168.0.10 "pip3 install flask pyserial"

# Step 3: Run application
ssh root@192.168.0.10 "cd /home/root/enersion/python && python3 web_app.py"

# Step 4: Access GUI
# Browser: http://192.168.0.10:8080

────────────────────────────────────────────────────────────

# Next Steps

## Immediate Tasks

**Install Python Dependencies on MYIR**

[`]bash

pip3 install flask pyserial

[`]

**Test RS485 Communication**

[`]bash

./scripts/test_rs485_myir.sh rx

[`]

**Run Web Application**

[`]bash

python3 web_app.py

[`]

**Verify DI/DO Communication**

Open browser to http://192.168.0.10:8080

Check connection status indicators

Test DI readings

Test DO control

## Future Enhancements

| Priority | Task | Description |
| --- | --- | --- |
| High | Analog Support | Add 4-20mA analog input reading |
| High | Auto-start Service | Systemd service configuration |
| Medium | Data Logging | Historical data storage |
| Medium | Alarms | Threshold-based alerts |
| Low | Multi-language | Turkish/English UI |
| Low | User Auth | Login/password protection |

────────────────────────────────────────────────────────────

# Known Issues

| Issue | Status | Workaround |
| --- | --- | --- |
| tkinter not available on MYIR | Fixed | Using Flask web UI |
| Qt build requires SDK | Open | Build on target board |

────────────────────────────────────────────────────────────

# Testing Checklist

## Hardware Tests

| Test | Command | Expected |
| --- | --- | --- |
| Ping MYIR | `ping 192.168.0.10` | Response OK |
| SSH Connection | `ssh root@192.168.0.10` | Login success |
| Serial Port | `ls /dev/ttySTM9` | File exists |
| GPIO Export | `cat /sys/class/gpio/gpio138/value` | 0 or 1 |

## Software Tests

| Test | Command | Expected |
| --- | --- | --- |
| Protocol CRC | Run unit tests | All pass |
| Ping DI | `./enersion_test ping 0x02` | Response |
| Ping DO | `./enersion_test ping 0x03` | Response |
| Read DI | `./enersion_test read di` | 8 bytes |
| Write DO | `./enersion_test write do` | ACK |

────────────────────────────────────────────────────────────

# Reference Information

## Device Addresses

| Address | Device | Function |
| --- | --- | --- |
| 0x00 | Broadcast | All devices |
| 0x01 | CTRL_420 | Analog inputs |
| 0x02 | CTRL_DIO | Digital inputs |
| 0x03 | CTRL_OUT | Digital outputs |
| 0x10 | Master | Linux HMI |

## RS485 Parameters

| Parameter | Value |
| --- | --- |
| Device | /dev/ttySTM9 |
| Baud Rate | 115200 |
| Data Bits | 8 |
| Parity | None |
| Stop Bits | 1 |
| GPIO | 138 (PI10) |

────────────────────────────────────────────────────────────

# Project Timeline

Week 1: Infrastructure ████████████████████ 100%
Week 2: Services       ████████████████████ 100%
Week 3: UI Development ████████████████████ 100%
Week 4: Testing        ████████████░░░░░░░░  60%
Week 5: Deployment     ░░░░░░░░░░░░░░░░░░░░   0%
        ─────────────────────────────────────────
Overall Progress:      ████████████████░░░░  80%

────────────────────────────────────────────────────────────

# Change Log

## Version 2.0 (December 16, 2024)

Added comprehensive project documentation

Created Hardware, Firmware, Software docs

Updated root README.md

Organized docs folder structure

## Version 1.0 (December 15, 2024)

Initial release

HAL layer complete

Protocol layer complete

Web GUI ready

Qt/QML code complete

────────────────────────────────────────────────────────────

*End of Document*